Cost of Charging for Alexandria Residents Without Off-Street Parking

Scenario Analysis for Residential EV Charging Access Policy

# Executive Summary

This short analysis compares three ways an Alexandria resident without off-street parking might charge an electric vehicle: 1) home charging through a permitted sidewalk crossing with an approved cable ramp; 2) routine Supercharging/DC fast charging; and 3) publicly accessible curbside Level 2 charging under Alexandria's curbside charging program.

The central result is that the cost difference is driven less by electricity and more by time. Using the base assumptions, home charging costs about $1,128 per year, including electricity, annualized charger and ramp costs, and an estimated annual insurance cost. Routine Supercharging costs about $6,637 per year, and public curbside Level 2 charging costs about $16,018 per year under the base case.

Even under a deliberately conservative stress test, home charging remains less expensive. If public Level 2 charging is assumed to be always available and all charging time is productive, it costs about $3,201 per year. If home charging is evaluated using the upper insurance estimate of $500 per year, it costs about $1,328 per year. The least-cost public-charging case is still about $1,873 more per year than the worst-cost home-charging case.

# Base-Case Comparison

| Scenario | Electricity / yr | Capital / yr | Insurance / yr | Time cost / yr | Total / yr | 5-year cost | Cost / mile |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Home charger + ramp | $468 | $360 | $300 | $0 | $1,128 | $5,640 | $0.09/mi |
| Supercharger / DC fast charging | $1,080 | $0 | $0 | $5,557 | $6,637 | $33,185 | $0.55/mi |
| Public curbside Level 2 charging | $720 | $0 | $0 | $15,298 | $16,018 | $80,090 | $1.33/mi |

# Model Inputs

| Input | Base value | Reason / source |
| --- | --- | --- |
| Ownership period | 5 years | Comparison period used for amortized capital costs |
| Annual driving | 12,000 miles | Round-number assumption for household EV use |
| EV energy use | 0.30 kWh/mile | Illustrative wall-to-wheels assumption for real-world Tesla use |
| Annual electricity use | 3,600 kWh | 12,000 miles x 0.30 kWh/mile |
| Value of time | $59.54/hour | Alexandria median household income of $123,840 divided by 2,080 work hours |
| Home electricity price | $0.13/kWh | Dominion bill estimated current cost |
| Supercharger price | $0.30/kWh | Estimated from web data |
| Public curbside L2 price | $0.20/kWh | Estimated from Seattle cost |
| Installed home charger cost | $1,500 | Amortized over five years (no discount rate) |
| Cable ramp cost | $300 | Added to capital cost and amortized over five years (no discount rate) |
| Annualized home capital cost | $360 | ($1,500 charger + $300 ramp) / 5 years |
| Annual insurance estimate | $300 base; $500 upper case | Assumed cost for $1,000,000 personal liability or umbrella coverage, depending on City requirements |

# Scenario Definitions

## Scenario 1: Home Charger with Sidewalk Crossing Ramp

The home scenario assumes a resident can charge from private electrical service using a home charger and an approved sidewalk crossing ramp or other approved cable-management device. The capital cost includes $1,500 for the installed charger and $300 for the ramp, amortized over five years. The base case also includes $300 per year for insurance. The upper home-cost case uses $500 per year for insurance.

## Scenario 2: Supercharger / DC Fast Charging

The Supercharger scenario assumes a routine 20 percent to 80 percent charging cycle of about 45 kWh. Each session requires about 20 minutes of travel each way plus 30 minutes of charging, or 70 minutes per session. At 3,600 kWh per year, this requires about 80 sessions per year.

## Scenario 3: Public Curbside Level 2 Charging

The public curbside Level 2 scenario assumes a 50 amp circuit delivering about 9.6 kW and a 3-hour charging session, or 28.8 kWh per session. At 3,600 kWh per year, this requires about 125 sessions per year. The base case assumes 10 minutes of travel each way, 50 percent productive charging time, and a 25 percent unavailability rate.

| Metric | Home charger + ramp | Supercharger | Public curbside L2 |
| --- | --- | --- | --- |
| Sessions per year | Not modeled | 80 | 125 |
| Energy per session | As needed | 45 kWh | 28.8 kWh |
| Nonproductive hours per year | 0 | 93.3 | 256.9 |
| Time cost per year | $0 | $5,557 | $15,298 |
| Electricity price | $0.13/kWh | $0.30/kWh | $0.20/kWh |
| Capital and insurance | $360 capital + $300 insurance | $0 | $0 |

# Home-Charging Insurance Sensitivity

The home-charging scenario is not very sensitive to the assumed insurance premium. Even if the annual insurance cost rises to the upper estimate of $500 per year, the total annual cost remains far below the public-charging scenarios.

| Insurance / yr | Electricity / yr | Capital / yr | Total / yr | 5-year cost | Cost / mile |
| --- | --- | --- | --- | --- | --- |
| $150 | $468 | $360 | $978 | $4,890 | $0.08/mi |
| $300 | $468 | $360 | $1,128 | $5,640 | $0.09/mi |
| $500 | $468 | $360 | $1,328 | $6,640 | $0.11/mi |

# Sensitivity Analysis: Public Curbside Level 2 Charging

Public Level 2 charging is most sensitive to how much charging time can be used productively and how often the charger is unavailable. The model uses the following formula:

Annual time cost = Sessions x [Travel time + Charging time x (1 - Productive fraction) + Failed-attempt time x Unavailability / (1 - Unavailability)] x Value of time

| Productive charging time | 0% unavailable | 10% unavailable | 25% unavailable | 40% unavailable |
| --- | --- | --- | --- | --- |
| 0% | $25,528 | $26,079 | $27,182 | $28,835 |
| 25% | $19,946 | $20,497 | $21,600 | $23,254 |
| 50% | $14,364 | $14,916 | $16,018 | $17,672 |
| 75% | $8,782 | $9,334 | $10,436 | $12,090 |
| 100% | $3,201 | $3,752 | $4,855 | $6,508 |

| Productive charging time | 0% unavailable | 10% unavailable | 25% unavailable | 40% unavailable |
| --- | --- | --- | --- | --- |
| 0% | 417 hrs | 426 hrs | 444 hrs | 472 hrs |
| 25% | 323 hrs | 332 hrs | 351 hrs | 378 hrs |
| 50% | 229 hrs | 238 hrs | 257 hrs | 285 hrs |
| 75% | 135 hrs | 145 hrs | 163 hrs | 191 hrs |
| 100% | 42 hrs | 51 hrs | 69 hrs | 97 hrs |

# Direct Stress Test: Best Public Case vs. Worst Home Case

The following comparison intentionally favors public charging and disfavors home charging. The public case assumes the charger is never unavailable and that 100 percent of the 3-hour Level 2 charging session can be used productively. The home case assumes the upper insurance estimate of $500 per year.

| Case | Electricity / yr | Capital + insurance / yr | Time cost / yr | Total / yr | 5-year cost | Cost / mile |
| --- | --- | --- | --- | --- | --- | --- |
| Best public Level 2 case | $720 | $0 | $2,481 | $3,201 | $16,004 | $0.27/mi |
| Worst home case | $468 | $360 + $500 insurance | $0 | $1,328 | $6,640 | $0.11/mi |
| Difference: public over home | $252 |  | $2,481 | $1,873 | $9,364 | $0.16/mi |

Result: Even under this public-favorable stress test, the least-cost public charging case costs about $1,873 more per year, or about $9,364 more over five years, than the worst-cost home charging case modeled here.

# Interpretation for Policy

The model illustrates why public charging should be treated as a complement to home charging, not a complete substitute. Public curbside charging is valuable for residents, visitors, renters, and locations where private charging is not feasible. But for residents who can safely charge from private electrical service, public charging can impose large recurring time costs.

The home sidewalk-crossing scenario has the lowest cost because it uses residential electricity and avoids repeated trip time. Adding a $300 cable ramp, a $300 annual insurance estimate, and the full installed charger cost still leaves home charging at about $1,128 per year in the base case.

The Supercharger scenario is much faster than public Level 2 charging, but it still imposes substantial time costs because it requires repeated trips away from home. In the base case, the time cost alone is about $5,557 per year.

The public curbside Level 2 scenario is especially sensitive to whether charging time is productive. If the resident can use all charging time productively and the charger is always available, annual cost falls to about $3,201. But under the base case of 50 percent productive time and 25 percent unavailability, the annual cost is about $16,018.

A residential EV sidewalk-crossing permit therefore has a clear economic rationale. It does not replace public curbside charging. It fills a distinct gap for residents who have access to private electrical service but lack off-street parking.

# Policy Conclusion

A limited residential EV sidewalk-crossing permit program would allow Alexandria to preserve pedestrian safety while reducing the economic penalty faced by residents without off-street parking. The annual cost of home charging remains low even after including the capital cost of a charger, the cost of a cable ramp, and a reasonable annual insurance premium. Public charging remains essential, but it is not an economic substitute for residential charging access where a safe permitted crossing is feasible.

# Sources and Notes

[1] FHWA Highway Statistics 2024, Table VM-1. The table reports 2024 average miles traveled per vehicle for all motor vehicles as 11,071. This analysis uses 12,000 miles as a round-number household driving assumption.

[2] Federal Reserve Bank of St. Louis / Census SAIPE, Estimate of Median Household Income for Alexandria City, VA. The 2024 estimate is $123,840. This analysis divides that amount by 2,080 work hours to estimate the hourly value of time.

[3] City of Alexandria, Electric Vehicles in Alexandria. The City states that Council approved a curbside EV charging program on February 21, 2026 to bring publicly accessible EV chargers to neighborhood streets at no cost to the City.

[4] A variety of assumptions that may need refining in this draft include: $0.13/kWh home electricity, $0.30/kWh Supercharger price, $0.20/kWh public Level 2 price, $1,500 installed home charger cost, $300 cable ramp cost, and the charging/travel/productive-time assumptions used in the scenarios.

[5] Insurance assumptions are illustrative. The base case uses $300 per year and the upper case uses $500 per year for personal liability or umbrella coverage, depending on what the City accepts. A requirement for commercial general liability or naming the City as an additional insured could produce different costs or availability constraints.