# Commercial Curbside Level 2 Charging in Alexandria, Virginia

Break-even pricing, electricity cost, and utilization analysis for public right-of-way operators

Working policy analysis | August 2026

| Baseline electricity cost | ~16.4¢/delivered kWh | Direct Dominion GS-1, 20% utilization |
| --- | --- | --- |
| Annual electricity spend | ~$2,190/port-year | 7.6 kW port, 20% utilization |

# Executive summary

Alexandria's curbside charging program can support commercially viable Level 2 charging, but utilization and installation cost dominate the economics. For a 7.6 kW curbside port served directly under Dominion Energy Virginia's small-general-service tariff, this analysis uses a rounded 14.0¢/metered-kWh electricity input. After an assumed 8% electrical loss and the $13.39 monthly GS-1 customer charge, electricity costs approximately 16.4¢ per delivered kWh at 20% utilization, or about $2,190 per charger per year. The theoretical high-utilization floor is about 15.2¢ per delivered kWh before the fixed customer charge.

The central commercial case is $7,500 installed cost per port, a five-year economic life, 10% required capital return, $1,250 per year in fixed operations and maintenance costs, and a 5% revenue-processing allowance. Under those assumptions, break-even retail pricing is approximately 52¢/kWh at 15% utilization, 43¢ at 20%, 38¢ at 25%, and 34¢ at 30%. A best-case $3,000 installation reduces break-even pricing to about 39¢, 33¢, 30¢, and 28¢ at the same utilization levels.

The policy implication is straightforward: once installed cost is reasonably low, site utilization matters more than small changes in permit fees or charger hardware. A city program that helps operators identify high-demand blocks, minimizes make-ready uncertainty, and allows inexpensive building-powered connections can materially improve economics.

# 1. Alexandria program context

Alexandria approved a curbside EV charging program in February 2026. Qualified vendors may install publicly accessible chargers in the public right-of-way at no cost to the City, and vendors are responsible for installation, maintenance, and customer service. The City's initial license term is five years, beginning after final inspection of each charger. The agreement also permits third-party site-host arrangements for electrical power, subject to recording and assignment requirements.

The City's insurance requirements are broader than a single $1 million liability policy. They include commercial general liability of $1 million per occurrence and $2 million aggregate, $1 million automobile liability, a $2 million umbrella, workers' compensation, and cyber coverage. For a scaled charging operator, these are treated here as portfolio-level costs allocated across ports rather than as a full stand-alone insurance premium charged to every charger.

# 2. Electricity cost

A single 7.6 kW curbside charger should ordinarily fit Dominion Energy Virginia Schedule GS-1, which applies to nonresidential customers that do not repeatedly reach 30 kW of peak demand. GS-1 has a fixed customer charge plus distribution, generation, transmission, and applicable rider charges. Generation rates vary by season and by whether monthly usage exceeds 1,400 kWh.

Modeling treatment. Because the filed tariff is a stack of base rates and changing riders rather than a single quoted commercial energy price, the analysis uses a rounded annualized input of 14.0¢ per metered kWh. This is a modeling value calibrated to the current GS-1 structure, not a quoted Dominion flat rate. The model adds the $13.39 monthly customer charge and assumes 8% losses between metered electricity and energy delivered to vehicles.

| Utilization | Delivered kWh/yr | Metered kWh/yr | Electricity $/yr | Electricity ¢/delivered kWh |
| --- | --- | --- | --- | --- |
| 10% | 6,658 | 7,237 | $1,174 | 17.6¢ |
| 15% | 9,986 | 10,855 | $1,680 | 16.8¢ |
| 20% | 13,315 | 14,473 | $2,187 | 16.4¢ |
| 25% | 16,644 | 18,091 | $2,693 | 16.2¢ |
| 30% | 19,973 | 21,710 | $3,200 | 16.0¢ |
| 40% | 26,630 | 28,946 | $4,213 | 15.8¢ |
| 50% | 33,288 | 36,183 | $5,226 | 15.7¢ |

Model calculation. Electricity cost includes the GS-1 single-phase customer charge, a 14.0¢/metered-kWh blended tariff input, and 8% electrical losses.

Top-line electricity benchmarks. At 20% utilization, one 7.6 kW port delivers about 13,315 kWh per year and incurs roughly $2,190 in electricity cost, equivalent to 16.4¢ per delivered kWh. At 30% utilization, annual electricity spending rises to about $3,200, but the unit cost falls to about 16.0¢/kWh as the fixed customer charge is spread over more energy. The underlying energy-only floor is approximately 15.2¢ per delivered kWh after losses.

# 3. Cost and finance assumptions

| Assumption | Central value | Rationale / sensitivity |
| --- | --- | --- |
| Port power | 7.6 kW | Matches the published specification for it's electric's curbside unit. |
| Economic life | 5 years | Matches Alexandria's initial license term. |
| Required return / capital cost | 10% | Used to annualize installed capital over five years. |
| Installed cost | $7,500/port | $3k best-case; $10k and $15k sensitivities. |
| Fixed O&M | $1,250/port-year | Includes maintenance, network/software, insurance allocation, permitting/admin, and support. |
| Revenue processing | 5% of revenue | Sensitivity can be run at 3% to 10%. |
| Electrical losses | 8% | Meter-to-vehicle loss assumption. |
| Direct electricity | 14.0¢/metered kWh | Rounded annualized GS-1 modeling input plus monthly customer charge. |
| Hosted-site payment | +6¢/delivered kWh | Observed property-owner revenue share published by it's electric. |

At a 10% required return over five years, the capital-recovery factor is 0.264. That converts installed costs of $3,000, $7,500, $10,000, and $15,000 into annual capital recovery of approximately $791, $1,978, $2,638, and $3,957 per port, respectively.

# 4. Break-even retail price: direct Dominion service

The table below gives the retail price required for annual revenue to cover electricity, fixed O&M, capital recovery, and the 5% revenue-processing allowance. It does not include income tax or a separate corporate profit margin beyond the 10% required return embedded in capital recovery.

| Utilization | Annual delivered kWh | $3k install | $7.5k install | $10k install | $15k install |
| --- | --- | --- | --- | --- | --- |
| 5% | 3,329 | $0.86 | $1.23 | $1.44 | $1.86 |
| 10% | 6,658 | $0.51 | $0.70 | $0.80 | $1.01 |
| 15% | 9,986 | $0.39 | $0.52 | $0.59 | $0.73 |
| 20% | 13,315 | $0.33 | $0.43 | $0.48 | $0.58 |
| 25% | 16,644 | $0.30 | $0.37 | $0.42 | $0.50 |
| 30% | 19,973 | $0.28 | $0.34 | $0.37 | $0.44 |
| 40% | 26,630 | $0.25 | $0.29 | $0.32 | $0.37 |
| 50% | 33,288 | $0.23 | $0.27 | $0.29 | $0.33 |

Central case. At $7,500 installed cost, break-even falls from about $0.52/kWh at 15% utilization to $0.43 at 20%, $0.38 at 25%, and $0.34 at 30%. At the $3,000 best-case installation cost, the corresponding values are approximately $0.39, $0.33, $0.30, and $0.28/kWh.

# 5. Break-even retail price: building-hosted service

A building-hosted architecture can reduce make-ready and utility-interconnection cost by using spare capacity in an adjacent building. It's electric publicly states that it separately meters this electricity, reimburses the property owner for electricity, and pays a 6¢/kWh revenue share. The table below therefore adds 6¢ per delivered kWh while omitting a separate new-service customer charge.

| Utilization | $3k install | $7.5k install | $10k install | $15k install |
| --- | --- | --- | --- | --- |
| 5% | $0.87 | $1.24 | $1.45 | $1.87 |
| 10% | $0.55 | $0.73 | $0.84 | $1.05 |
| 15% | $0.44 | $0.56 | $0.63 | $0.77 |
| 20% | $0.38 | $0.48 | $0.53 | $0.63 |
| 25% | $0.35 | $0.43 | $0.47 | $0.55 |
| 30% | $0.33 | $0.39 | $0.43 | $0.50 |
| 40% | $0.30 | $0.35 | $0.38 | $0.43 |
| 50% | $0.29 | $0.33 | $0.35 | $0.39 |

Interpretation. At equal installed cost, direct Dominion service is cheaper because there is no host revenue share. The hosted model is attractive when it materially reduces installation cost. For example, at 20% utilization a $3,000 hosted installation breaks even around $0.38/kWh, compared with about $0.43/kWh for a $7,500 direct-service installation.

# 6. Utilization is the critical commercial variable

The economics are highly nonlinear at low utilization because fixed costs are spread over very little energy. At 5% utilization, even a $3,000 direct-service installation requires about $0.86/kWh. Between 15% and 30% utilization, however, break-even cost falls rapidly.

New York City's early curbside Level 2 pilot provides a useful real-world benchmark. Overall plug-in utilization reached 34% in December 2022; the ten busiest sites exceeded 54%, while the ten least-busy sites were below 22%. NYC reported in 2026 that its remaining curbside network was occupied by an EV more than 70% of the time. Alexandria should not assume New York utilization, but the data show that 20% to 30% utilization is plausible for well-selected urban curbside sites.

# 7. Policy implications for Alexandria

• Site selection matters more than modest permit-fee changes. At 20% utilization, moving from a $15,000 installation to $7,500 lowers break-even price by roughly 16¢/kWh, while a small one-time permit fee would have a much smaller effect.

• Make-ready risk should be treated as a core program-design issue. Building-powered installations can trade a higher variable electricity cost for much lower interconnection and construction cost.

• A five-year license creates real capital-recovery pressure. Longer successor terms, predictable renewals, or lower removal risk would reduce the annual revenue required to recover installation cost.

• The insurance package is material but should be evaluated at the operator-fleet level. Requiring each charger to economically bear a full stand-alone policy premium would overstate actual commercial cost for a scaled operator.

• The most useful performance data for Alexandria's first two years will be utilization, delivered kWh, uptime, installation cost, maintenance incidents, and operator-reported cost categories. Those data can replace several of the assumptions in this analysis before the City evaluates program expansion.

# Conclusion

For a small Alexandria curbside Level 2 charger, electricity itself is likely to cost the operator on the order of 16¢ per delivered kWh, with a reference annual electricity bill of about $2,200 per charger at 20% utilization. That establishes a meaningful floor beneath retail charging prices. Above that floor, commercial viability is primarily a function of installation cost, fixed operating overhead, and utilization.

Under the central $7,500 installed-cost case, retail pricing in the mid-30s to low-40s cents per kWh becomes viable around 20% to 30% utilization. Under a true best-case $3,000 installation, the same price range becomes viable closer to 15% to 20% utilization. The evidence therefore supports a program strategy centered on low-cost make-ready and high-quality site selection rather than assuming that charger hardware or electricity alone will determine commercial success.

# References

1. City of Alexandria. Electric Vehicles in Alexandria. Updated April 15, 2026. https://www.alexandriava.gov/energy/electric-vehicles-in-alexandria

2. City of Alexandria. Curbside EV Charging License Agreement and Program Requirements. February 2026. https://www.alexandriava.gov/sites/default/files/2026-02/EV%20Curbside%20Charging%20License%20Agreement.pdf

3. Dominion Energy Virginia. Schedule GS-1, Small General Service. Effective for usage on and after January 1, 2026. https://www.dominionenergy.com/-/media/content/rates-and-tariffs/pdfs/virginia/large-business/schedule-gs1.pdf

4. Dominion Energy Virginia. Exhibit of Applicable Riders. Effective May 1, 2026. https://www.dominionenergy.com/-/media/content/rates-and-tariffs/pdfs/virginia/shared/exhibit-of-applicable-riders.pdf

5. U.S. Department of Energy, Alternative Fuels Data Center. Operation and Maintenance for Electric Vehicle Charging Infrastructure. https://afdc.energy.gov/fuels/electricity-infrastructure-maintenance-and-operation

6. it's electric. For Buildings. Published specifications and property-host revenue share. https://itselectric.us/for-buildings

7. New York City Department of Transportation. Curbside Level 2 EV Charging Pilot: Evaluation Report. 2023. https://www.nyc.gov/html/dot/downloads/pdf/curbside-level-2-charging-pilot-evaluation-report.pdf

8. New York City Department of Transportation. Expansion of Curbside Electric Vehicle Charging Across Five Boroughs. August 2026. https://www.nyc.gov/html/dot/html/pr2026/expansion-of-curbside-electric-vehicle-charging.shtml

Important modeling note: The 14.0¢/metered-kWh Dominion input is a rounded analytical assumption derived from the GS-1 tariff structure and current riders. It is not a quoted flat Dominion rate. Actual bills will vary by billing month, usage block, applicable riders, taxes, service configuration, and future tariff changes.